Experiment No:1

Title: Study and implementation of basic arithmetic and loop operations using 8086 Assembly

Language.

Aim: To write and execute 8086 assembly language programs for basic arithmetic operations

and number series calculations.

Software / Apparatus Required

- Computer System

- 8086 Emulator / EMU8086 / DOSBox / VS Code

- Assembler and debugger

Theory

The 8086 microprocessor supports arithmetic and logical operations using instructions such as:

- ADD – Adds two operands

- SUB – Subtracts one operand from another

- MUL – Multiplies two numbers

- DIV – Divides two numbers

- INC / DEC – Increment or decrement value

- LOOP / JNZ – Used for repetition of instructions

These instructions help perform mathematical calculations and iterative operations in assembly

language programs.

Write Algorithm and 8086 Assembly Language programs to perform basic arithmetic operations and number-series calculations such as:

- Find the sum of single-digit numbers.

Algorithm:

- Start

- Input 2 numbers and store them in registers AL and BL

- Add the numbers

- Store the result in AL

- Convert the result into ASCII

- Display the result

- Stop

Code:

.MODEL SMALL

.STACK 100H

.DATA

.CODE

MAIN PROC

MOV AX,@DATA

MOV DS,AX

MOV AL,3

MOV BL,4

ADD AL,BL

ADD AL,30H

MOV DL,AL

MOV AH,02H

INT 21H

MOV AH,4CH

INT 21H

MAIN ENDP

END MAIN

Output:

7

- Find the sum of double-digit numbers.

Algorithm:

- Start

- Input two numbers and store them in AL and BL

- Add the numbers

- Store the result in AL

- Separate the digits using AAM

- Convert digits into ASCII

- Display the result

- Stop

Code:

.MODEL SMALL

.STACK 100H

.DATA

.CODE

MAIN PROC

MOV AX,@DATA

MOV DS,AX

MOV AL,15

MOV BL,12

ADD AL,BL

MOV AH,0

MOV BL,10

DIV BL

ADD AL,30H

MOV DL,AL

MOV AH,02H

INT 21H

MOV AL,AH

ADD AL,30H

MOV DL,AL

MOV AH,02H

INT 21H

MOV AH,4CH

INT 21H

MAIN ENDP

END MAIN

Output:

22

- Perform subtraction of single-digit numbers.

Algorithm:

- Start

- Input two numbers and store them in AL and BL

- Subtract the second number from the first number

- Store result in AL

- Convert result into ASCII

- Display result

- Stop

Code:

.MODEL SMALL

.STACK 100H

.DATA

.CODE

MAIN PROC

MOV AX,@DATA

MOV DS,AX

MOV AL,9

MOV BL,5

SUB AL,BL

ADD AL,30H

MOV DL,AL

MOV AH,02H

INT 21H

MOV AH,4CH

INT 21H

MAIN ENDP

END MAIN

Output:

4

- Perform subtraction of double-digit numbers.

Algorithm:

- Start

- Input two numbers and store them in AL and BL

- Subtract the numbers

- Store result in AL

- Convert result into ASCII

- Display result

- Stop

Code:

.MODEL SMALL

.STACK 100H

.DATA

.CODE

MAIN PROC

MOV AX,@DATA

MOV DS,AX

MOV AL,25

MOV BL,13

SUB AL,BL

MOV AH,0

MOV BL,10

DIV BL

ADD AL,30H

MOV DL,AL

MOV AH,02H

INT 21H

MOV AL,AH

ADD AL,30H

MOV DL,AL

MOV AH,02H

INT 21H

MOV AH,4CH

INT 21H

MAIN ENDP

END MAIN

Output:

12

- Perform multiplication of single-digit numbers.

Algorithm:

- Start

- Input two numbers and store them in AL and BL

- Multiply the numbers

- Store result

- Convert result into ASCII

- Display result

- Stop

Code:

.MODEL SMALL

.STACK 100H

.DATA

.CODE

MAIN PROC

MOV AX,@DATA

MOV DS,AX

MOV AL,3

MOV BL,2

MUL BL

ADD AL,30H

MOV DL,AL

MOV AH,02H

INT 21H

MOV AH,4CH

INT 21H

MAIN ENDP

END MAIN

Output:

6

- Perform multiplication of double-digit numbers.

Algorithm:

- Start

- Input two numbers and store them in AL and BL

- Multiply the numbers

- Store result

- Convert result into ASCII

- Display result

- Stop

Code:

.MODEL SMALL

.STACK 100H

.DATA

.CODE

MAIN PROC

MOV AX,@DATA

MOV DS,AX

MOV AL,16

MOV BL,2

MUL BL

MOV BL,10

DIV BL

MOV DL,AL

ADD DL,30H

MOV AH,02H

INT 21H

MOV DL,AH

ADD DL,30H

MOV AH,02H

INT 21H

MOV AH,4CH

INT 21H

MAIN ENDP

END MAIN

Output:

32

- Perform division of single-digit numbers.

Algorithm:

- Start

- Input two numbers and store them in registers AL and BL

- Divide the first number by the second number

- Store the result in AL

- Convert the result into ASCII

- Display the result

- Stop

Code:

.MODEL SMALL

.STACK 100H

.DATA

.CODE

MAIN PROC

MOV AX,@DATA

MOV DS,AX

MOV AL,8

MOV BL,2

MOV AH,0

DIV BL

ADD AL,30H

MOV DL,AL

MOV AH,02H

INT 21H

MOV AH,4CH

INT 21H

MAIN ENDP

END MAIN

Output:

4

- Perform division of double-digit numbers.

Algorithm:

- Start

- Input two numbers and store them in AL and BL

- Divide the numbers

- Store the result

- Convert result into ASCII

- Display the result

- Stop

Code:

.MODEL SMALL

.STACK 100H

.DATA

.CODE

MAIN PROC

MOV AX,@DATA

MOV DS,AX

MOV AL,20

MOV BL,4

MOV AH,0

DIV BL

ADD AL,30H

MOV DL,AL

MOV AH,02H

INT 21H

MOV AH,4CH

INT 21H

MAIN ENDP

END MAIN

Output:

5

- Find the sum of first n natural numbers.

Algorithm:

- Start

- Initialize N

- Initialize sum as 0

- Add numbers from 1 to N

- Store result

- Convert result into ASCII

- Display result

- Stop

Code:

.MODEL SMALL

.STACK 100H

.DATA

N DB 5

.CODE

MAIN PROC

MOV AX,@DATA

MOV DS,AX

MOV CL,N

MOV BL,1

MOV AL,0

L1:

ADD AL,BL

INC BL

DEC CL

JNZ L1

MOV AH,0

MOV BL,10

DIV BL

MOV BH,AH

ADD AL,30H

MOV DL,AL

MOV AH,02H

INT 21H

MOV DL,BH

ADD DL,30H

MOV AH,02H

INT 21H

MOV AH,4CH

INT 21H

MAIN ENDP

END MAIN

Output:

15

- Find the sum of even numbers up to n.

Algorithm:

- Start

- Initialize N

- Start from number 2

- Add even numbers up to N

- Store result

- Convert result into ASCII

- Display result

- Stop

Code:

.MODEL SMALL

.STACK 100H

.DATA

N DB 6

.CODE

MAIN PROC

MOV AX,@DATA

MOV DS,AX

MOV AL,0

MOV BL,2

MOV CL,N

L1:

ADD AL,BL

ADD BL,2

CMP BL,CL

JBE L1

MOV AH,0

MOV BL,10

DIV BL

MOV BH,AH

ADD AL,30H

MOV DL,AL

MOV AH,02H

INT 21H

MOV DL,BH

ADD DL,30H

MOV AH,02H

INT 21H

MOV AH,4CH

INT 21H

MAIN ENDP

END MAIN

Output:

20

- Find the sum of odd numbers up to n.

Algorithm:

- Start

- Initialize N

- Start from number 1

- Add odd numbers up to N

- Store result

- Convert result into ASCII

- Display result

- Stop

Code:

.MODEL SMALL

.STACK 100H

.DATA

N DB 5

.CODE

MAIN PROC

MOV AX,@DATA

MOV DS,AX

MOV AL,0

MOV BL,1

MOV CL,N

L1:

ADD AL,BL

ADD BL,2

CMP BL,CL

JBE L1

ADD AL,30H

MOV DL,AL

MOV AH,02H

INT 21H

MOV AH,4CH

INT 21H

MAIN ENDP

END MAIN

Output:

25

- Find the sum of squares of first n numbers (1² + 2² + … + n²).

Algorithm:

- Start

- Initialize N

- Initialize sum as 0

- Multiply each number by itself

- Add the result to sum

- Repeat until N numbers

- Convert result to ASCII

- Display result

- Stop

Code:

.MODEL SMALL

.STACK 100H

.DATA

N DB 3

.CODE

MAIN PROC

MOV AX,@DATA

MOV DS,AX

MOV CL,N

MOV BL,1

MOV AL,0

L1:

MOV DL,BL

MOV AH,0

MOV AL,BL

MUL DL

ADD AL,BH

MOV BH,AL

INC BL

DEC CL

JNZ L1

MOV AL,BH

MOV AH,0

MOV BL,10

DIV BL

MOV BH,AH

ADD AL,30H

MOV DL,AL

MOV AH,02H

INT 21H

MOV DL,BH

ADD DL,30H

MOV AH,02H

INT 21H

MOV AH,4CH

INT 21H

MAIN ENDP

END MAIN

Output:

14

- Find the sum of cubes of first n numbers (1³ + 2³ + … + n³).

Algorithm:

- Start

- Initialize N

- Initialize sum as 0

- Take numbers from 1 to N

- Find cube of each number

- Add the cube to the sum

- Convert result into ASCII

- Display the result

- Stop

Code:

.MODEL SMALL

.STACK 100H

.DATA

N DB 3

.CODE

MAIN PROC

MOV AX,@DATA

MOV DS,AX

MOV CL,N

MOV BL,1

MOV BH,0

L1:

MOV AL,BL

MUL BL

MUL BL

ADD BH,AL

INC BL

DEC CL

JNZ L1

MOV AL,BH

MOV AH,0

MOV BL,10

DIV BL

MOV BH,AH

ADD AL,30H

MOV DL,AL

MOV AH,02H

INT 21H

MOV DL,BH

ADD DL,30H

MOV AH,02H

INT 21H

MOV AH,4CH

INT 21H

MAIN ENDP

END MAIN

Output:

36

- Find the sum of numbers from 1 to 100.

Algorithm:

- Start

- Initialize sum as 0

- Start from number 1

- Add numbers up to 100

- Store result

- Convert result into ASCII

- Display result

- Stop

Code:

.MODEL SMALL

.STACK 100H

.DATA

.CODE

MAIN PROC

MOV AX,@DATA

MOV DS,AX

MOV CX,100

MOV AX,0

MOV BX,1

L1:

ADD AX,BX

INC BX

LOOP L1

MOV BX,10

MOV CX,0

L2:

MOV DX,0

DIV BX

PUSH DX

INC CX

CMP AX,0

JNE L2

L3:

POP DX

ADD DL,30H

MOV AH,02H

INT 21H

LOOP L3

MOV AH,4CH

INT 21H

MAIN ENDP

END MAIN

Output:

5050

- Find the sum of digits of a given number.

Algorithm:

- Start

- Initialize a number

- Separate the digits of the number

- Add the digits

- Store result

- Convert result to ASCII

- Display result

- Stop

Code:

.MODEL SMALL

.STACK 100H

.DATA

NUM DB 45

.CODE

MAIN PROC

MOV AX,@DATA

MOV DS,AX

MOV AL,NUM

MOV AH,0

MOV BL,10

DIV BL

ADD AL,AH

ADD AL,30H

MOV DL,AL

MOV AH,02H

INT 21H

MOV AH,4CH

INT 21H

MAIN ENDP

END MAIN

Output:

9

- Find the factorial of a number.

Algorithm:

- Start

- Initialize number N

- Initialize result as 1

- Multiply numbers from 1 to N

- Store result

- Convert result to ASCII

- Display result

- Stop

Code:

.MODEL SMALL

.STACK 100H

.DATA

N DB 5

.CODE

MAIN PROC

MOV AX,@DATA

MOV DS,AX

MOV AL,1

MOV BL,1

MOV CL,N

L1:

MUL BL

INC BL

DEC CL

JNZ L1

MOV BX,10

MOV CX,0

L2:

MOV DX,0

DIV BX

PUSH DX

INC CX

CMP AX,0

JNE L2

L3:

POP DX

ADD DL,30H

MOV AH,02H

INT 21H

LOOP L3

MOV AH,4CH

INT 21H

MAIN ENDP

END MAIN

Output:

120

- Generate the Fibonacci series up to n terms.

Algorithm:

- Start

- Initialize first two numbers of series

- Generate next numbers by adding previous two numbers

- Repeat until N terms

- Display the numbers

- Stop

Code:

.MODEL SMALL

.STACK 100H

.DATA

N DB 5

.CODE

MAIN PROC

MOV AX,@DATA

MOV DS,AX

MOV CL,N

MOV AL,0

MOV BL,1

L1:

ADD AL,BL

ADD AL,30H

MOV DL,AL

MOV AH,02H

INT 21H

SUB AL,30H

MOV DL,BL

MOV BL,AL

MOV AL,DL

DEC CL

JNZ L1

MOV AH,4CH

INT 21H

MAIN ENDP

END MAIN

Output:

12358

- Calculate the average of n numbers.

Algorithm:

- Start

- Initialize N numbers

- Find the sum of numbers

- Divide the sum by N

- Convert result into ASCII

- Display the result

- Stop

Code:

.MODEL SMALL

.STACK 100H

.DATA

SUM DB 20

N DB 4

.CODE

MAIN PROC

MOV AX,@DATA

MOV DS,AX

MOV AL,SUM

MOV AH,0

MOV BL,N

DIV BL

ADD AL,30H

MOV DL,AL

MOV AH,02H

INT 21H

MOV AH,4CH

INT 21H

MAIN ENDP

END MAIN

Output:

5

Conclusion

In this experiment, I learned how to write and execute basic programs in 8086 assembly language. I understood how to perform arithmetic operations like addition, subtraction, multiplication, and division using different instructions. I also learned how loops can be used to perform repetitive calculations such as finding the sum of numbers, factorial, Fibonacci series, and average. Through this experiment, I became familiar with registers, basic instructions, and displaying output in assembly language.