Experiment No:2                                                                                 Date: 12/03/2026

Title: Study and implementation of number analysis and series generation using 8086 Assembly Language.

Aim: To write and execute 8086 assembly language programs for counting even and odd numbers, checking palindrome numbers, counting positive and negative numbers, and generating number series.

Software / Apparatus Required

- Computer System

- 8086 Emulator / EMU8086 / DOSBox / VS Code

- Assembler and debugger

Theory

The 8086 microprocessor performs decision-making and iterative operations using conditional

and looping instructions. The following concepts are used:

- DIV instruction – Used to check even/odd numbers (remainder check)

- CMP instruction – Compares two values

- JZ / JNZ – Conditional jumps based on comparison

- LOOP instruction – Repeats a set of instructions

- INT 21H – Used for input/output operations

Applications:
• Even numbers → divisible by 2
• Palindrome → number remains same after reversing
• Positive/Negative → based on sign
• Series generation → repeated addition using loop

Write Algorithm and 8086 Assembly Language programs to perform basic arithmetic operations and number-series calculations such as:

- Write an 8086 assembly language program to count the number of even and odd numbers from a given set of numbers.

Algorithm:

- Start

- Initialize array and count

- Set even = 0, odd = 0

- Check each element

- If divisible by 2 → even++

- Else → odd++

- Display counts

- Stop

Program:

.MODEL SMALL

.STACK 100H

.DATA

ARR DB 1,2,3,4,5

N   DB 5

.CODE

MAIN PROC

MOV AX, @DATA

MOV DS, AX

MOV SI, 0

MOV CX, 0

MOV CL, N

MOV BL, 0

MOV BH, 0

LOOP1:

MOV AL, ARR[SI]

MOV AH, 0

MOV DL, 2

DIV DL

CMP AH, 0

JZ EVEN

INC BH

JMP NEXT

EVEN:

INC BL

NEXT:

INC SI

LOOP LOOP1

MOV DL, BL

ADD DL, 30H

MOV AH, 02H

INT 21H

MOV DL, ' '

INT 21H

MOV DL, BH

ADD DL, 30H

MOV AH, 02H

INT 21H

MOV AH, 4CH

INT 21H

MAIN ENDP

END MAIN

Output:

2  3

- Write an 8086 assembly language program to check whether a given number is a palindrome or not.

Algorithm:

- Start

- Initialize number

- Store original number

- Reverse the number

- Compare reversed with original

- If equal → palindrome

- Else → not palindrome

- Stop

Program:

.MODEL SMALL

.STACK 100H

.DATA

NUM DW 121

.CODE

MAIN PROC

MOV AX, @DATA

MOV DS, AX

MOV AX, NUM

MOV BX, 10

MOV CX, AX        ; store original number

MOV SI, 0         ; SI = reversed number

REVERSE:

MOV DX, 0

DIV BX            ; AX / 10

MOV DI, DX        ; last digit

MOV AX, SI

MUL BX            ; reverse * 10

ADD AX, DI        ; + digit

MOV SI, AX        ; update reverse

MOV AX, CX

MOV DX, 0

DIV BX

MOV CX, AX        ; update remaining number

CMP CX, 0

JNZ REVERSE

MOV AX, NUM

CMP AX, SI

JZ PAL

MOV DL, 'N'

MOV AH, 02H

INT 21H

JMP EXIT

PAL:

MOV DL, 'Y'

MOV AH, 02H

INT 21H

EXIT:

MOV AH, 4CH

INT 21H

MAIN ENDP

END MAIN

Output:

Y

- Write an 8086 assembly language program to count the number of positive and negative numbers from a given set of numbers.

Algorithm:

- Start

- Initialize array

- Set positive = 0, negative = 0

- Check each element

- If ≥ 0 → positive++

- Else → negative++

- Display counts

- Stop

Program:

.MODEL SMALL

.STACK 100H

.DATA

ARR DB -3, 4, -2, 5, 1

N   DB 5

.CODE

MAIN PROC

MOV AX, @DATA

MOV DS, AX

MOV SI, 0

MOV CX, 0

MOV CL, N

MOV BL, 0    ; positive count

MOV BH, 0    ; negative count

LOOP1:

MOV AL, ARR[SI]

CMP AL, 0

JL NEGATIVE

INC BL

JMP NEXT

NEGATIVE:

INC BH

NEXT:

INC SI

LOOP LOOP1

; print positive count

MOV DL, BL

ADD DL, 30H

MOV AH, 02H

INT 21H

MOV DL, ' '

INT 21H

; print negative count

MOV DL, BH

ADD DL, 30H

MOV AH, 02H

INT 21H

MOV AH, 4CH

INT 21H

MAIN ENDP

END MAIN

Output:

3  2

- Write an 8086 assembly language program to generate a series like 1, 3, 5, 7, … up to n terms

Algorithm:

- Start

- Initialize n

- Set number = 1

- Display number

- Add 2 to number

- Repeat until n terms

- Stop

Program:

.MODEL SMALL

.STACK 100H

.DATA

N DB 5

.CODE

MAIN PROC

MOV AX, @DATA

MOV DS, AX

MOV CX, 0

MOV CL, N

MOV BL, 1

LOOP1:

MOV DL, BL

ADD DL, 30H

MOV AH, 02H

INT 21H

MOV DL, ' '

MOV AH, 02H

INT 21H

ADD BL, 2

LOOP LOOP1

MOV AH, 4CH

INT 21H

MAIN ENDP

END MAIN

Output:

1  3  5  7  9

- Write an 8086 assembly language program to find the largest number in a given set of numbers.

Algorithm:

- Start

- Initialize array

- Assume first element as largest

- Compare with each element

- Update if larger found

- Display largest

- Stop

Program:

.MODEL SMALL

.STACK 100H

.DATA

ARR DB 2, 9, 4, 7, 5

N   DB 5

.CODE

MAIN PROC

MOV AX, @DATA

MOV DS, AX

MOV SI, 0

MOV CX, 0

MOV CL, N

MOV AL, ARR[SI]   ; assume first element is largest

INC SI

DEC CL

L1:

MOV BL, ARR[SI]

CMP BL, AL

JLE NEXT

MOV AL, BL        ; update largest

NEXT:

INC SI

LOOP L1

MOV DL, AL

ADD DL, 30H

MOV AH, 02H

INT 21H

MOV AH, 4CH

INT 21H

MAIN ENDP

END MAIN

Output:

9

- Write an 8086 assembly language program to find the smallest number in a given set of numbers.

Algorithm:

- Start

- Initialize array

- Assume first element as smallest

- Compare with each element

- Update if smaller found

- Display smallest

- Stop

Program:

.MODEL SMALL

.STACK 100H

.DATA

ARR DB 2,9,4,7,5

N   DB 5

.CODE

MAIN PROC

MOV AX, @DATA

MOV DS, AX

MOV SI, 0

MOV CX, 0

MOV CL, N

MOV AL, ARR[SI]

INC SI

DEC CL

L1:

MOV BL, ARR[SI]

CMP BL, AL

JGE NEXT

MOV AL, BL

NEXT:

INC SI

LOOP L1

MOV DL, AL

ADD DL, 30H

MOV AH, 02H

INT 21H

MOV AH, 4CH

INT 21H

MAIN ENDP

END MAIN

Output:

2

- Write an 8086 assembly language program to search an element in an array using linear search.

Algorithm:

- Start

- Initialize array and key

- Compare key with each element

- If match found → success

- Else continue

- If not found → failure

- Stop

Program:

.MODEL SMALL

.STACK 100H

.DATA

ARR DB 2,4,6,8,5

N   DB 5

KEY DB 6

.CODE

MAIN PROC

MOV AX, @DATA

MOV DS, AX

MOV SI, 0

MOV CX, 0

MOV CL, N

MOV AL, KEY

SEARCH:

MOV BL, ARR[SI]

CMP BL, AL

JZ FOUND

INC SI

LOOP SEARCH

MOV DL, 'N'

MOV AH, 02H

INT 21H

JMP EXIT

FOUND:

MOV DL, 'Y'

MOV AH, 02H

INT 21H

EXIT:

MOV AH, 4CH

INT 21H

MAIN ENDP

END MAIN

Output:

Y

- Write an 8086 assembly language program to sort a set of numbers in ascending order.

Algorithm:

- Start

- Initialize array

- Compare adjacent elements

- Swap if needed

- Repeat for all elements

- Display sorted array

- Stop

Program:

.MODEL SMALL

.STACK 100H

.DATA

ARR DB 5,2,8,1,4

N   DB 5

.CODE

MAIN PROC

MOV AX, @DATA

MOV DS, AX

MOV CL, N

DEC CL

OUTER:

MOV SI, 0

MOV CH, CL

INNER:

MOV AL, ARR[SI]

MOV BL, ARR[SI+1]

CMP AL, BL

JBE NEXT

MOV ARR[SI], BL

MOV ARR[SI+1], AL

NEXT:

INC SI

DEC CH

JNZ INNER

LOOP OUTER

MOV SI, 0

MOV CL, N

PRINT:

MOV DL, ARR[SI]

ADD DL, 30H

MOV AH, 02H

INT 21H

MOV DL, ' '

MOV AH, 02H

INT 21H

INC SI

LOOP PRINT

MOV AH, 4CH

INT 21H

MAIN ENDP

END MAIN

Output:

1  2  4  5  8

Conclusion

The experiment was successfully carried out to study number analysis and series generation using 8086 Assembly Language. Programs such as counting even and odd numbers, checking palindrome numbers, counting positive and negative numbers, searching, sorting, and generating series were implemented correctly. The use of conditional and looping instructions like CMP, JZ, JNZ, and LOOP enhanced understanding of decision-making and control flow in assembly language. This experiment strengthened the knowledge of data processing and logical operations in the 8086 microprocessor.